General Industry Threshold Heights Requiring Fall Prevention/Protection

| #### Condition | #### Threshold | #### Method | Standard and Rule |
| --- | --- | --- | --- |
| Over dangerous equipment (includes vats, tanks, electrical equipment, machinery, equipment or machinery with protruding parts) | < 4 feet >4 feet | Guardrail, travel restraint, or equipment guarded to eliminate the hazard Guardrail system, safety net, travel restraint, personal fall arrest system | Part 2. Walking-Working Surfaces, Rule 1910.28(b)(6) |
| Unprotected Edge | 4 feet | Guardrail system, safety net, personal fall arrest system | Part 2. Walking-Working Surfaces, 1910.28(b)(1) |
| Vehicle servicing pit | <10 feet | Limited access to authorized/trained workers, contrasting markings 6 feet from edge, caution signs | Part 2. Walking-Working Surfaces, 1910.28(b)(8) |
| Holes (including skylights) | 4 feet | Covers, Guardrail system, travel restraint, personal fall arrest system | Part 2. Walking-Working Surfaces, Rule 1910.28(b)(3) |
| Wall opening (with or without chute) | 4 feet | Guardrail system, safety net, travel restraint, personal fall arrest system | Part 2. Walking-Working Surfaces, 1910.28(b)(7) |
| Fixed ladder | < 24 feet >24 feet | None Required Installed prior to 11/19/2018-Personal fall arrest system, ladder safety system, cage or well New installed after 11/19/2018- Personal fall arrest system or ladder safety system All fixed ladders on or after 11/1//2036- Personal fall arrest system or ladder safety system | Part 2. Walking-Working Surfaces, 1910.28(b)(9) |
| Portable ladder |  | None Required |  |
| Scaffold Rope Descent System | 10 feet 4 feet | Guardrail system, safety net, travel restraint, personal fall arrest system Personal fall arrest system | Part 2. Walking-Working Surfaces, 1910.28(12) |
| Low-slope roof | < 6 feet from edge >6 feet < 15 feet >15 feet | Guardrail system, safety net, travel restraint, personal fall arrest system Guardrail system, safety net, travel restraint, personal fall arrest system, or use a designated area system if work is infrequent and temporary Not required if infrequent and temporary, but must implement and enforce rule | Part 2. Walking-Working Surfaces, 1910.28(13) |
| Stationary refuse packer hopper opening | No minimum | Guardrail | Part 17. Refuse Packer Units, Rule 1732(1) |
| Overhead crane, footwalk or catwalk | No minimum | Guardrail | Part 18. Overhead & Gantry Cranes, Rule 1835(2) |
| Personnel platforms - overhead, underhung, crawler cranes, powered industrial trucks, tractors | No minimum | Guardrail | Part 18, Part 19, Part 20, Part 21 Part 22 |
| Underhung crane catwalk | No minimum | Guardrail | Part 20. Underhung Cranes & Monorail Systems, Rule 2016(3) |
| Manlifts | 25 feet No minimum | Guardrail (emergency landing) Entrances and exits to manlifts | Part 25. Manlifts, Rules 1910.68 (b)(6)(v) and 1910.68 (b)(8) |

This chart provides a breakdown of the fall protection requirements of general industry standards. Check to see if specific rules relate to your industry or activities. It's important that you look at the specific language in the standard, which can be found by clicking on the hyperlink.

| ##### Condition | #### Threshold | #### Method | Standard and Rule |
| --- | --- | --- | --- |
| #### When no specific rule applies - climbing on machinery or equipment not equipped with a platform and standard barrier | #### 4 feet | #### Guardrail system, travel restraint, personal fall arrest system | Part 2. Walking-Working Surfaces, Rule 1910.28(b)(15) |
| At or near rolls of board hammer | No minimum | A fixed or portable platform equipped with guardrail system. | Part 42. Forging, Rule 4249(2) |
| Crucible or furnace platform, melting furnace pit Non-ferrous crucible/furnace | No minimum No minimum | Guardrail system. Grating or tailing | Part 44. Foundries, Rules 4436(3), 4451(b), 4493(1) |
| Open bin, bunker, hopper | <36 inches above working level | Grated opening or guardrail system | Part 52. Sawmills, Rule 5229(1) |
| Aerial lift | No minimum | Personal Fall Arrest System (follow lift manufacturers specifications for fall protection) | Part 58. Aerial Work Platforms, Rule 5836(1)(2) |
| Aloft in a tree | No minimum | Safety belt, tree-trimming saddle belt, rope saddle, safety strap | Part 53. Tree Trimming & Removal, Rule 5331(1) |
| Spray equipment from a moving vehicle | No minimum | Seat belt, guardrail system | Part 53. Tree Trimming & Removal, Rule 5344 |
| Oil rig floor or other surfaces | 10 feet | Safety belt or harness and lanyard (fall restraint) | Part 57. Oil and Gas Drilling and Servicing Operations, Rule 5713 |
| Mounting machinery * Plastic molding only | No minimum | Platform or ladder only | Part 62. Plastic Molding, Rule 6223 |
| Catwalk (Pulp and paper) Catwalk – dangerous equipment | 4 feet No minimum | Guardrail system Guardrail system | Part 63. Pulp, Paper and Paperboard Mills, Rule 6322(5) |
| Flume or water runway used to transport logs | No minimum | Guardrail system | Part 63. Pulp, Paper and Paperboard Mills, Rule 6343(1) |
| Inclined walkway along jack ladder | No minimum | Stair rail | Part 63. Pulp, Paper and Paperboard Mills, Rule 6343(2) |
| Barker walkway | No minimum | Guardrail system | Part 63. Pulp, Paper and Paperboard Mills, Rule 6346(3)(4) |
| Pulper vessel, beater tub | Opening < 42 inches from floor or work platform | Guardrail system | Part 63. Pulp, Paper and Paperboard Mills, Rule 6361(1) |
| Bleacher, opening & platform | No minimum | Guardrail system | Part 63. Pulp, Paper and Paperboard Mills, Rule 6362 |
| Aerial apparatus-firefighting | No minimum | Safety belt system | Part 74. Fire Fighting, Rule 7424(2) |
| Stand on or in stored grain Receiving pits | No minimum | Body harness & lifeline with observer and communication Grating with 2.5-inch maximum opening | Part 77. Grain Handling Facilities, Rule 1910.272(g)(2) & (g)(3) |
| Working above ground in tree | No minimum | Climbing rope & safety saddle | Part 86. Electric Power Generation, Transmission & Distribution, 1910.269(r)(8) |
| Cover removed – enclosed space | No minimum | Railing, temporary cover, or other barrier | Part 86. continued, 1910.269(e)(5) |
| Elevated locations i.e. poles, towers, similar structures | More than 4 feet above ground | Fall arrest equipment, work positioning equipment or travel restricting equipment with 6 feet free fall limit | Part 86. continued, 1910.269(g)(2)(iv)(C)(3) (see Notes 1 & 2) |
| Cover removed-confined space | No minimum | Railing, temporary cover or barrier | Part 90. Confined Space Entry, 1910.146(c)(5)(ii)(B) |

Contact:  	Consultation Education and Training (CET) Division

Michigan Occupational Safety and Health Administration (MIOSHA)

517-284-7720 or www.michigan.gov/miosha

MIOSHA/CET #0171 (Revised 01/14/20)